# Strategic Site Selection Analysis

## Lower Mississippi River: Optimal Zones for Deep-Water + Pipeline Access

Geographic Scope: Below Baton Rouge (~Mile 230 AHP) to Head of Passes (Mile 0 AHP) Focus: Sites combining deep-water vessel access with proximity to major pipeline systems Purpose: Industrial terminal or pipeline interconnection site development

## Site Selection Criteria

### Critical Factors:

- Deep-water access - Draft capability for mid-sized to large vessels (25+ ft draft)

- Pipeline proximity - Within economical lateral construction distance (<15 miles ideal, <30 miles acceptable)

- Pipeline system significance - Proximity to major transmission systems or convergence points

- Industrial zoning - Existing or achievable industrial land use designation

- Multimodal access - Rail, truck, barge capability

- Labor availability - Workforce proximity

- Existing infrastructure - Utilities, roads, developed parcels

- Environmental factors - Wetland avoidance, hurricane risk, floodplain considerations

## TIER 1 ZONES - Optimal Combined Access

### 🏆 Zone 1A: St. James Parish (East Bank) - Convent/Gramercy/Wallace Area

River Mile: 160-170 AHP

Why This is THE Premier Location: - MAJOR PIPELINE CONVERGENCE POINT - unmatched in lower Mississippi corridor - Deep-water access: Excellent (upriver of New Orleans congestion, downriver of Baton Rouge) - Industrial establishment: Mature industrial corridor

Pipeline Infrastructure Density:

Crude Oil (HIGHEST DENSITY): - ✅ Capline Pipeline Terminus - 1.2 million BPD, 40-inch diameter, from Patoka IL - Plains All American operated - Delivers to St. James terminal complex - Serves Marathon Garyville, Shell Norco, Phillips 66 Alliance refineries

- ✅ Bayou Bridge Pipeline Terminus - 480,000 BPD, 24-inch diameter

  - Energy Transfer operated

  - Nederland TX to St. James

  - Connects to St. James terminal infrastructure

Natural Gas (Strong Likelihood): - Gulf South Pipeline (Boardwalk) - Extensive Louisiana intrastate network likely passes through/near - Transco Zone 3 (Williams) - Major interstate system, possible lateral access - Tennessee Gas 500 Line (Kinder Morgan) - Louisiana routing, potential proximity

Refined Products: - Plantation Pipeline origin (Baton Rouge ~60 miles upriver, but St. James in distribution zone) - Multiple refinery product loading terminals

Connected Facilities: - Marathon Garyville Refinery (578,000 BPD) - ~15 miles upriver (West Bank) - Shell Norco Refinery (~50,000 BPD processing) - ~10 miles downriver (West Bank) - Phillips 66 Alliance Refinery (250,000 BPD) - ~20 miles downriver (West Bank) - Multiple crude oil storage terminals (St. James is THE Louisiana crude hub)

Site Selection Advantages: - Existing terminal infrastructure: Multiple crude terminals, marine docks, tank farms - Pipeline convergence: Two major crude systems terminate here (rare combination) - Rail access: Class I rail service (likely CN, UP access) - Industrial zoning: Fully established industrial corridor - Labor pool: St. James/Ascension/Baton Rouge workforce accessible - Property availability: Industrial parcels likely available (check St. James Parish GIS)

Optimal Use Cases: - Crude oil storage/blending terminal (primary opportunity - leverages Capline + Bayou Bridge) - Natural gas storage interconnection (if pipeline lateral to Gulf South/Transco feasible) - Construction materials terminal (less optimal than closer to New Orleans, but serviceable) - Refined products distribution (refinery cluster proximity)

Challenges: - High competition (established industrial area, may have limited available parcels) - Petroleum focus may limit diversification options

Search Strategy: - Target parcels: 100-500 acres with river frontage in St. James Parish industrial zone - GIS resources: St. James Parish assessor, Louisiana State GIS - Proximity target: <5 miles from Capline/Bayou Bridge terminus

Verdict: ⭐⭐⭐⭐⭐ If crude oil infrastructure is the priority, this is THE location

### 🏆 Zone 1B: Plaquemines Parish (West Bank) - Braithwaite/Belle Chasse Area

River Mile: 75-85 AHP

Why AMAX Current Site is Already Optimal: - Strategic middle position: Between New Orleans industrial demand and Plaquemines LNG - Deep-water access: Excellent (80 AHP is well-positioned for vessel traffic) - Multimodal advantage: Rare combination of river + Class I rail + extensive laydown

Pipeline Infrastructure:

Crude Oil (EXCELLENT): - ✅ Harvest CAM Pipeline System - <5 miles estimated (HIGH CONFIDENCE) - Harvest Midstream operated - West Bank crude oil gathering system - Serves New Orleans area refineries: - Phillips 66 Alliance (250k BPD, ~20 mi upriver) - PBF Chalmette (190k BPD, ~15 mi across river) - Marathon Garyville (578k BPD, ~45 mi upriver) - Connects to Belle Chasse Terminal (3,200 acres, 1M bbl storage, ~10 mi upriver)

Natural Gas (GOOD to EXCELLENT): - Gulf South Pipeline (Boardwalk) - 10-25 miles estimated (MODERATE CONFIDENCE) - Extensive Louisiana intrastate network - Flexibility for storage interconnection (no FERC interstate approval required) - Likely routing through/near Plaquemines Parish given Louisiana coverage

- Gator Express Pipeline (Venture Global) - Location TBD, potentially 20-50 miles

  - Feeds Plaquemines LNG (50-75 miles south in same parish)

  - 2.9 Bcf/d capacity

  - Same parish as AMAX = potential interconnection opportunity

- Transco Zone 3 (Williams) - 20-50 miles estimated (LOW CONFIDENCE without GIS)

  - Major interstate system

  - Requires lateral construction + FERC approval

Refined Products (Reference): - Colonial Lines 1 & 2 - 15-30 miles (East Bank) - Collins Pipeline - 15-20 miles (East Bank, Chalmette terminus)

NGL (Reference): - Cayenne Pipeline - 10-20 miles to river crossing (Venice to Toca route)

Connected Facilities: - Belle Chasse Terminal (Harvest, ~10 mi upriver) - Competitive comparison; demonstrates West Bank crude terminal viability - Phillips 66 Alliance Refinery (~20 mi upriver, West Bank) - Plaquemines LNG (~50-75 mi south, same parish) - 2.9 Bcf/d Venture Global facility

AMAX Site Specific Advantages: - ✅ 387 acres - Substantial footprint for terminal or storage development - ✅ Norfolk Southern Class I rail + reciprocal switching to all Class I carriers - ✅ 386,000 SF laydown yard - Rare for Gulf Coast sites - ✅ Deep-draft dock (510’ x 52’ + 774’ mooring) - ✅ 341 acres upland (only 45 acres wetland per 2014 USACE JD) - ✅ West Bank location - Less industrial congestion than East Bank - ✅ Existing infrastructure - Minimizes greenfield capital investment

Optimal Use Cases: - Natural gas storage facility (Case A) - Gulf South interconnection or Transco lateral; load balancing for Plaquemines LNG - Construction materials terminal (Case B) - Multimodal logistics for LNG/data center construction materials - Crude oil storage/distribution - Harvest CAM proximity enables crude terminal operations - LNG-related infrastructure support - Plaquemines LNG proximity

Challenges: - Gulf South interconnection requires confirmation of exact routing/distance - Gas storage requires geological feasibility study (salt dome presence) - Plaquemines Parish workforce smaller than upriver locations

Search Strategy: - AMAX site is already here - 387 acres at Braithwaite - Alternative parcels: Search Plaquemines Parish West Bank (Belle Chasse to Braithwaite corridor) for 100-300 acre sites with river frontage - Competitive comparison: Belle Chasse Terminal sets precedent for West Bank crude operations

Verdict: ⭐⭐⭐⭐⭐ AMAX current location is optimal for multimodal + gas storage development

### 🏆 Zone 1C: St. Charles Parish (West Bank) - Norco/Destrehan/Hahnville Area

River Mile: 130-140 AHP

Strategic Position: - Between Baton Rouge and New Orleans - Central to Louisiana industrial corridor - Deep-water access: Excellent - Marathon Garyville proximity: Immediate (578,000 BPD refinery complex)

Pipeline Infrastructure:

Crude Oil (EXCELLENT): - Marathon Garyville Refinery receives crude via multiple systems: - Capline (terminates St. James, ~20 mi upriver, but Marathon is major delivery point) - Bayou Bridge (terminates St. James) - Crude oil gathering systems serving Marathon complex

Natural Gas (EXCELLENT): - Highly likely Gulf South Pipeline proximity - Marathon refinery is major gas consumer - Transco potential - Major industrial area likely served by interstate transmission - Tennessee Gas potential - Louisiana routing crosses northern parishes; laterals possible

Refined Products (GOOD): - Colonial Lines 1 & 2 likely nearby (East Bank corridor, but may have West Bank crossings or laterals) - Marathon Garyville produces refined products requiring pipeline egress

Connected Facilities: - Marathon Garyville Refinery Complex (578,000 BPD combined Garyville and Norco operations) - Shell Norco Refinery adjacent - Valero St. Charles Refinery nearby

Site Selection Advantages: - Refinery cluster proximity - Three major refineries within 5-10 miles - Natural gas demand density - Refineries consume substantial gas (process heating, hydrogen production) - Pipeline corridor certainty - Marathon complex ensures gas transmission infrastructure - Labor pool: Strong (St. Charles/Jefferson Parish workforce) - Industrial zoning: Established corridor - Rail access: Class I rail likely (Union Pacific, Canadian National)

Optimal Use Cases: - Natural gas storage interconnection - Marathon refinery gas demand creates strong market - Refined products terminal - Refinery cluster proximity - Crude oil terminal - Capline/Bayou Bridge delivery to Marathon creates opportunity - Industrial gas distribution - Hydrogen, process gas for refineries

Challenges: - High industrial density may limit available parcels - Petroleum focus may limit diversification

Search Strategy: - Target area: St. Charles Parish West Bank between Norco and Hahnville - Parcel size: 100-400 acres with river frontage - Focus: West Bank parcels adjacent to Marathon Garyville infrastructure corridor - GIS resources: St. Charles Parish assessor, Louisiana State GIS

Verdict: ⭐⭐⭐⭐⭐ Optimal for gas storage serving refinery cluster; strong pipeline + deep water combination

## TIER 2 ZONES - Strong Pipeline + Water Access

### Zone 2A: Plaquemines Parish (East Bank) - Chalmette/Meraux/Arabi Area

River Mile: 90-105 AHP (St. Bernard Parish transitions to Plaquemines)

Pipeline Infrastructure:

Crude Oil (GOOD): - PBF Chalmette Refinery (190,000 BPD) receives crude via: - Harvest CAM system (from West Bank via crossing or indirect routing) - MOEM Pipeline system - Barge deliveries

Refined Products (EXCELLENT): - ✅ Collins Pipeline Terminus - 16-inch, 125 miles to Collins MS - PBF 80%, Valero 20% ownership - Originates at Chalmette Refinery - Delivers gasoline, jet fuel, diesel to Mississippi - Connects to Plantation and Colonial systems

Natural Gas (GOOD): - Gulf South Pipeline likely (PBF Chalmette refinery gas demand) - Transco potential (East Bank corridor)

Connected Facilities: - PBF Chalmette Refinery (190,000 BPD) - Collins Pipeline origin point - Refinery product terminals

Site Selection Advantages: - Collins Pipeline origin - Refined products infrastructure - Refinery proximity - PBF Chalmette major facility - East Bank location - Colonial/Collins pipeline corridor - New Orleans metro proximity - Workforce, services, logistics - Industrial zoning - Established St. Bernard Parish industrial area

Optimal Use Cases: - Refined products storage/distribution - Collins Pipeline origin advantage - Refinery support services - Turnaround materials, maintenance logistics - Construction materials terminal - New Orleans area demand (closer than AMAX)

Challenges: - St. Bernard Parish has limited available river-front industrial parcels (established area) - PBF Chalmette dominates immediate area - East Bank congestion (more developed than West Bank)

Search Strategy: - Target area: St. Bernard/Plaquemines Parish boundary, East Bank - Parcel size: 50-200 acres (smaller parcels likely given developed area) - Focus: Meraux/Violet area downriver from Chalmette Refinery

Verdict: ⭐⭐⭐⭐ Strong for refined products infrastructure; Collins Pipeline origin is unique advantage

### Zone 2B: Ascension Parish (West Bank) - Donaldsonville Area

River Mile: 175-185 AHP

Pipeline Infrastructure:

Natural Gas (EXCELLENT): - CF Industries Ammonia Plant - Major natural gas consumer (ammonia production) - Gulf South Pipeline certain (serves CF Industries) - Interstate systems likely (major industrial gas demand)

NGL (EXCELLENT): - ✅ Cayenne Pipeline Terminus - Venice to Toca - 40,000-50,000+ BPD Y-grade NGLs - Toca Gas Processing/Fractionation Facility - Crosses Mississippi River ~10-20 miles south

Crude Oil (MODERATE): - Capline passes through region (Patoka IL to St. James LA) - Bayou Bridge passes through region

Connected Facilities: - Toca Gas Processing Plant (Targa Resources) - Cayenne Pipeline destination - CF Industries Donaldsonville - Major ammonia production (natural gas consumer) - Multiple chemical plants

Site Selection Advantages: - Cayenne Pipeline terminus - NGL fractionation hub - CF Industries gas demand - Natural gas infrastructure certainty - Chemical corridor - Established industrial area - Deep-water access: Good - Labor pool: Ascension Parish + Baton Rouge proximity

Optimal Use Cases: - NGL storage/distribution - Cayenne terminus proximity - Natural gas storage - CF Industries demand + Cayenne gas processing creates strong market - Chemical feedstock terminal - Petrochemical corridor positioning

Challenges: - Industrial density may limit available parcels - Focused on petrochemicals/ammonia (less diversification)

Search Strategy: - Target area: Ascension Parish West Bank near Donaldsonville - Focus: Vicinity of Toca plant and CF Industries complex - Parcel size: 100-300 acres

Verdict: ⭐⭐⭐⭐ Excellent for NGL/gas storage; Cayenne terminus + CF Industries demand = strong pipeline access

### Zone 2C: Jefferson Parish (West Bank) - Harvey/Westwego Area

River Mile: 105-115 AHP

Pipeline Infrastructure:

Crude Oil (GOOD to EXCELLENT): - Harvest CAM system proximity (serves Phillips 66 Alliance Refinery ~10-15 mi upriver) - Multiple crude systems serving New Orleans refinery area

Natural Gas (GOOD): - Gulf South Pipeline likely (New Orleans metro area coverage) - Industrial demand in Harvey/Westwego area

Refined Products (GOOD): - Proximity to Colonial corridor (East Bank across river) - Phillips 66 Alliance refined products distribution

Connected Facilities: - Port of New Orleans - Adjacent (Harvey/Westwego is part of Port of NO complex) - Phillips 66 Alliance Refinery - ~10-15 miles upriver (West Bank) - Industrial/maritime corridor

Site Selection Advantages: - Port of New Orleans proximity - Established maritime infrastructure, logistics - New Orleans metro workforce - Large labor pool - Industrial zoning - Established area - Multimodal access - Port, rail (multiple Class I carriers), truck, river - Market proximity - New Orleans construction materials demand (Case B advantage)

Optimal Use Cases: - Construction materials terminal (PRIMARY) - New Orleans area demand, LNG construction materials logistics - Crude oil/products terminal - Port of NO integration - Industrial support logistics - Refinery turnaround materials, equipment staging

Challenges: - High industrial density (limited available parcels) - Port of New Orleans competition for river frontage - Urban/industrial congestion

Search Strategy: - Target area: Jefferson Parish West Bank (Harvey/Westwego/Marrero) - Parcel size: 50-200 acres (smaller parcels more realistic given density) - Focus: West Bank of Mississippi adjacent to Port of NO facilities

Verdict: ⭐⭐⭐⭐ Strong for construction materials terminal (Case B); New Orleans market proximity advantage

## TIER 3 ZONES - Specialized Opportunities

### Zone 3A: Plaquemines Parish (Both Banks) - Port Sulphur Area

River Mile: 50-65 AHP

Pipeline Infrastructure:

Natural Gas (EXCELLENT for LNG-related): - Gator Express Pipeline (Venture Global) - Proximity to Plaquemines LNG - Plaquemines LNG ~20-30 miles south - 2.9 Bcf/d capacity - Potential interconnection opportunity

- Gulf South Pipeline possible (southern Plaquemines Parish coverage)

Connected Facilities: - Port of Plaquemines - Deep-draft port facilities - Plaquemines LNG (~20-30 miles south) - 2.9 Bcf/d Venture Global facility

Site Selection Advantages: - Deeper water - Closer to passes, can handle larger vessels - Port of Plaquemines established - Deep-draft infrastructure - Plaquemines LNG proximity - LNG-related opportunities (load balancing storage, materials supply) - Gator Express potential - Same pipeline corridor as major LNG facility

Optimal Use Cases: - Natural gas storage for LNG load balancing - Plaquemines LNG proximity (20-30 mi vs. 50-75 mi from AMAX) - LNG construction materials terminal - Plaquemines LNG maintenance/expansion materials - Deep-draft cargo terminal - Vessel size capability

Challenges: - Remote location - Smaller workforce, limited services - Hurricane exposure - High risk (closer to Gulf, lower elevation) - Infrastructure gaps - Less developed than upriver areas - Limited rail access - May not have Class I rail service

Search Strategy: - Target area: Port Sulphur vicinity, both banks - Parcel size: 100-500+ acres (more greenfield options likely) - Focus: Port of Plaquemines adjacent parcels

Verdict: ⭐⭐⭐ Specialized opportunity for LNG-related infrastructure; deeper water but more remote

### Zone 3B: St. John the Baptist Parish (West Bank) - LaPlace/Edgard Area

River Mile: 145-155 AHP

Pipeline Infrastructure:

Natural Gas (MODERATE to GOOD): - Gulf South Pipeline likely (Louisiana coverage between Baton Rouge and New Orleans) - Interstate systems possible (Transco, Tennessee Gas)

Crude Oil (MODERATE): - Between St. James terminus and New Orleans refinery area - Potential pipeline corridor crossing

Site Selection Advantages: - Strategic position - Between two major industrial zones (Baton Rouge refineries and New Orleans refineries) - Deep-water access: Good - I-10 proximity - Major interstate highway - Rail access: Class I rail (likely CN, UP)

Optimal Use Cases: - Natural gas storage - Between major demand centers - Logistics hub - Mid-point between Baton Rouge and New Orleans - Construction materials distribution - Central position

Challenges: - Less pipeline density than St. James or Plaquemines/St. Bernard areas - Less established industrial corridor - May lack specific pipeline convergence advantages

Search Strategy: - Target area: St. John the Baptist Parish West Bank - Parcel size: 100-400 acres - GIS resources: St. John Parish assessor

Verdict: ⭐⭐⭐ Moderate opportunity; strategic position but less pipeline density than Tier 1 zones

### Zone 3C: Plaquemines Parish - Venice Area

River Mile: 10-20 AHP

Pipeline Infrastructure:

Natural Gas (MODERATE): - Gator Express proximity (Plaquemines LNG nearby) - Offshore gas gathering systems

NGL (EXCELLENT): - ✅ Cayenne Pipeline Origin - Venice Gas Processing Plant - 40,000-50,000+ BPD to Toca - Y-grade NGL gathering from offshore production

Connected Facilities: - Venice Gas Processing Plant (processes offshore gas production) - Offshore service base - Helicopters, crew boats, supply vessels - Plaquemines LNG - Nearby (~30-40 miles north)

Site Selection Advantages: - Deepest water - Closest to Gulf of Mexico, can handle largest vessels - Offshore industry hub - Established offshore oil/gas service infrastructure - Cayenne Pipeline origin - NGL infrastructure

Optimal Use Cases: - Offshore support base - Oil/gas service logistics - Deep-draft specialty cargo - Vessel size no constraint - NGL-related infrastructure - Cayenne Pipeline proximity

Challenges: - Extremely remote - Very limited workforce, services - Highest hurricane risk - Closest to Gulf, lowest elevation, most exposed - Limited infrastructure - Minimal road access, no rail - Environmental sensitivity - Critical wetlands, bird habitat (Audubon areas)

Search Strategy: - Not recommended for major industrial terminal development due to remoteness and hurricane risk - Suitable for: Offshore service base, specialized deep-draft operations only

Verdict: ⭐⭐ Specialized niche only; too remote and high-risk for major terminal development

## COMPARATIVE ANALYSIS MATRIX

| Zone | River Mile | Deep Water | Pipeline Density | Rail Access | Labor Pool | Industrial Zoning | Overall Score |
| --- | --- | --- | --- | --- | --- | --- | --- |
| St. James (East) | 160-170 | ⭐⭐⭐⭐⭐ | ⭐⭐⭐⭐⭐ (Crude hub) | ⭐⭐⭐⭐ | ⭐⭐⭐⭐ | ⭐⭐⭐⭐⭐ | ⭐⭐⭐⭐⭐ |
| AMAX / Braithwaite (West) | 75-85 | ⭐⭐⭐⭐⭐ | ⭐⭐⭐⭐ (Gas + crude) | ⭐⭐⭐⭐⭐ | ⭐⭐⭐ | ⭐⭐⭐⭐ | ⭐⭐⭐⭐⭐ |
| St. Charles/Norco (West) | 130-140 | ⭐⭐⭐⭐⭐ | ⭐⭐⭐⭐⭐ (Refinery) | ⭐⭐⭐⭐ | ⭐⭐⭐⭐⭐ | ⭐⭐⭐⭐⭐ | ⭐⭐⭐⭐⭐ |
| Chalmette (East) | 90-105 | ⭐⭐⭐⭐⭐ | ⭐⭐⭐⭐ (Products) | ⭐⭐⭐ | ⭐⭐⭐⭐ | ⭐⭐⭐⭐ | ⭐⭐⭐⭐ |
| Donaldsonville (West) | 175-185 | ⭐⭐⭐⭐ | ⭐⭐⭐⭐ (NGL + gas) | ⭐⭐⭐⭐ | ⭐⭐⭐⭐ | ⭐⭐⭐⭐ | ⭐⭐⭐⭐ |
| Harvey/Westwego (West) | 105-115 | ⭐⭐⭐⭐ | ⭐⭐⭐ | ⭐⭐⭐⭐ | ⭐⭐⭐⭐⭐ | ⭐⭐⭐⭐ | ⭐⭐⭐⭐ |
| Port Sulphur | 50-65 | ⭐⭐⭐⭐⭐ | ⭐⭐⭐ (LNG-related) | ⭐⭐ | ⭐⭐ | ⭐⭐⭐ | ⭐⭐⭐ |
| LaPlace/Edgard | 145-155 | ⭐⭐⭐⭐ | ⭐⭐⭐ | ⭐⭐⭐⭐ | ⭐⭐⭐ | ⭐⭐⭐ | ⭐⭐⭐ |
| Venice | 10-20 | ⭐⭐⭐⭐⭐ | ⭐⭐ (Offshore) | ⭐ | ⭐ | ⭐⭐ | ⭐⭐ |

## STRATEGIC RECOMMENDATIONS

### For Different Development Scenarios:

#### Crude Oil Storage/Distribution Terminal:

🥇 #1 Choice: St. James Parish (East Bank) - Capline + Bayou Bridge convergence = unmatched crude infrastructure - Established terminal corridor - Refinery cluster proximity

🥈 #2 Choice: AMAX / Braithwaite (West Bank) - Harvest CAM proximity (<5 mi) - Belle Chasse Terminal competitive comparison - Multimodal advantage (rail + river)

#### Natural Gas Storage Facility:

🥇 #1 Choice: AMAX / Braithwaite (West Bank) - Gulf South interconnection potential (10-25 mi) - Plaquemines LNG load balancing market (50-75 mi) - Existing 387-acre site with multimodal access

🥈 #2 Choice: St. Charles/Norco (West Bank) - Marathon refinery cluster gas demand - Pipeline corridor certainty - Strong market fundamentals

🥉 #3 Choice: Ascension/Donaldsonville (West Bank) - CF Industries major gas demand - Cayenne Pipeline gas processing infrastructure - Chemical corridor positioning

#### Construction Materials Terminal (Case B):

🥇 #1 Choice: Jefferson Parish / Harvey-Westwego (West Bank) - Closest to New Orleans demand center - Port of New Orleans integration - Strong multimodal access

🥈 #2 Choice: AMAX / Braithwaite (West Bank) - Norfolk Southern Class I rail + reciprocal switching - 386,000 SF laydown yard - Strategic position between New Orleans and LNG construction zones

🥉 #3 Choice: Chalmette/Meraux (East Bank) - New Orleans metro proximity - Refinery turnaround materials market - Collins Pipeline industrial corridor

#### LNG-Related Infrastructure:

🥇 #1 Choice: Port Sulphur Area - Closest to Plaquemines LNG (20-30 mi vs. 50-75 mi from AMAX) - Gator Express pipeline corridor - Port of Plaquemines deep-draft access

🥈 #2 Choice: AMAX / Braithwaite - Same parish as Plaquemines LNG - Multimodal access advantage - Gator Express potential proximity

## PROPERTY SEARCH STRATEGY

### Immediate Actions:

- GIS Analysis:

  - St. James Parish assessor GIS (parcel search, industrial zoning)

  - Plaquemines Parish assessor GIS (alternatives to AMAX)

  - St. Charles Parish assessor GIS (Norco/Destrehan area)

  - Louisiana State GIS (pipeline overlay with parcels)

- Pipeline Verification:

  - Louisiana Natural Gas Pipelines Feature Service overlay

  - Gulf South Pipeline route confirmation (contact Boardwalk)

  - Harvest CAM route confirmation (contact Harvest Midstream)

  - Capline/Bayou Bridge St. James terminus mapping

- Parcel Identification:

  - Target criteria: 100-500 acres, river frontage, industrial zoning (actual or achievable)

  - Priority parishes: St. James (East), Plaquemines (West, alternatives to AMAX), St. Charles (West)

  - Pipeline proximity verification: <15 miles to major system ideal

- Market Validation:

  - Pipeline operator discussions (interconnection feasibility)

  - Industrial real estate brokers (Louisiana Gulf Coast specialists)

  - Economic development agencies (St. James, Plaquemines, St. Charles parishes)

## CONCLUSION

The AMAX Port Nickel site (Braithwaite, Plaquemines Parish, West Bank, Mile 80 AHP) is ALREADY in an optimal location for deep-water + pipeline access. It ranks among the top 3 zones in the entire Baton Rouge-to-Head of Passes corridor.

If seeking alternative sites: - For crude oil focus: St. James Parish (East Bank) is THE premier location - Capline + Bayou Bridge convergence is unmatched - For gas storage/multimodal: AMAX current location or St. Charles/Norco area (West Bank) are optimal - For construction materials (Case B): Jefferson Parish/Harvey-Westwego has New Orleans market proximity advantage, but AMAX multimodal capability (especially rail) is competitive

Strategic insight: Rather than seeking alternative locations, AMAX site’s optimal positioning suggests development at the current location is the highest-value strategy. The combination of deep-water access, Harvest CAM crude proximity, Gulf South gas potential, Norfolk Southern rail, and 387-acre footprint is difficult to replicate elsewhere in the corridor.

Last Updated: February 4, 2026 Purpose: Strategic site selection analysis for deep-water + pipeline access optimization